| National University of Computer and Emerging Sciences, Lahore Campus |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Course: | Analog and Digital Communications | Course Code: | EE3003 |
|  | Program: | BS(Electrical Engineering) | Semester: | Fall 2024 |
|  | Exam: | Assignment 1 | Page(s): | 2 |
|  | Chapter(s): | 2,3 | Section: | ALL |
|  | Submission Date: |  | CLO: | 02 |
|  | Instructor(s): | Dr. S. M. Sajid, Mohsin Yousuf | Marks: | 40 |

Question#1 [CLO 02]										[05]

Check whether the following signals are energy or power signal and find there corresponding energy/power.

- xt=e-3t

- xt=cos2ωot

- xt=ut

- xt=12 (cosωt+1),                   -πω≤t≤πω0,                                            otherwise

Figure 1.d

- g1t=3cost+cos5t-2π3+cos8t+2π3

Question # 2 [CLO 02]										[05]

For the signal

gt=4at2+a2

Determine the essential bandwidth B Hz of g(t) such that the energy contained in the spectral components of g(t) of frequencies below B Hz is 92% of the signal energy Eg.

Hint: Determine Gω by applying the symmetry property.

Question # 3 [CLO 02]										[10]

Figure 2 below shows a random binary pulse train g(t). The pulse width is Tb/2, and one binary digit is transmitted every Tb seconds. A binary 1 is transmitted by the positive pulse, and a binary 0 is transmitted by the negative pulse. The two symbols are equally likely and occur randomly. Determine i) the autocorrelation function Rx(τ), ii) the PSD Sx(ω) and iii) the essential bandwidth of this signal.

Figure 2. Random Binary Pulse Train

Question # 4 [CLO 02]										[10]

The random binary signal x(t) shown in Figure 3 below transmits one digit every Tb seconds. A binary 1 is transmitted by a pulse p(t) of width Tb/2 and amplitude B; a binary 0 is transmitted by no pulse. The digits 1 and 0 are equally likely and occur randomly. Determine the i) autocorrelation function Rx(τ) and ii) the PSD Sx(ω).

Figure 3. Random Binary Signal

Question # 5 [CLO 02]										[10]

Find the mean square value (or power) of the output voltage y(t) of the system shown in figure 4 below if the input voltage PSD Sxω=2 rectω2. Calculate the power (mean square value) of the input signal x(t).

Figure 4. Circuit using differential system