# Complex Engineering Problem

Course Code and Title: EE2001 – Electronic Circuit Design

Semester: Fall 2024

Instructor: Dr. Omer Saleem

Total Marks: 50

Deadline: Tuesday, 10th Dec. 2024

# CLOs and PLOs for Complex Engineering Problem

| CLOs |  | Description | Taxonomy Levels | PLO |
| --- | --- | --- | --- | --- |
| CLO 2 | Theory | Design electronic circuits using op-amps. | C5 | 3 |

# Instructions

Attempt all parts of the following design problem.

# Problem Statement

Design a biomedical signal conditioning system to process and analyze Electromyogram (EMG) signals using operational amplifiers. EMG signals, typically in the range of 0.1 mV to 5 mV and 10 Hz to 500 Hz, are used to study the bicep muscle activity. The circuit should include multiple stages to amplify and condition the raw EMG signal while enabling feature extraction for diagnostic or control purposes.

- The system uses two EMG sensors. One EMG sensor measures the muscle activity of elbow. It delivers the reference voltage signal Vref. The other EMG sensor measures the muscle activity of bicep and delivers Vsig. The difference of these signals is instrumentally amplified by a gain of [your batch number] to acquire a differential EMG signal, VD.

- The differential EMG signal, VD, is passed through a customized circuit that integrates VD to compute the total muscle activity over time, and also differentiates it to detect rapid changes in muscle activity. Their combined effect is unified via the following expression.

VE=k1VEdt+k2dVEdt+k3

where, k1=first two digits of your 4-digit roll number

k2=last two digits of your 4-digit roll number

k3=sum of all digits of your 4-digit roll number

- Finally, the refined muscle activity signals VE is squared to compute the power of the EMG signal, Vo=VE2, for energy-based analysis.

# Objectives

- Design an op-amp circuit to implement all the intermediate stages of signal conditional discussed above (starting from Stage (a) to Stage (c)). Your design should include schematic diagrams for each stage with detailed annotations labeling each stage.

- Evaluate all the resistors, capacitors, and other circuit components used in the circuit. Show complete mathematical calculations to support your choice.

- State the issues (or imperfections) linked with the circuits in the intermediate stages and suggest plausible remedies.

- Operate within a limited power budget suitable for portable or wearable medical devices.

# Deliverables

- Schematic diagrams for each stage with detailed annotations and theoretical analysis for the evaluation of circuit components

- Design Report discussing the design strategy, choice of op-amp principles, challenges, and final performance metrics. Your report should include following points:

- Address the challenge of modular design while maintaining system integration.

- Provide proper justifications of each op-amp circuit configuration used to fulfill the desired objectives for the circuits in each stage.

- Comment on the alternative circuit designs for a given stage.

- Discuss the impact of tolerances of the components on the output voltage

# Complex Engineering Problem Attributes

| WP1: Depth of knowledge WP2: Range of conflicting requirements WP3: Depth of analysis WP4: Familiarity of issues WP5: Extent of applicable codes WP6: Extent of stakeholders WP7: Interdependence | Please fill according to the WPs covered in the course CEP, example is shown here. - WP1: Depth of Knowledge – Analysis and choice of op-amp principle for each stage requires knowledge of WK3, WK8. Design requires knowledge of WK5, WK8 - WP2: Range of conflicting requirements -- Requirements are conflicting. All objectives are desired, but power consumption should be minimized and DC imperfections must be dealt with. The initial amplification stage design is also critical. - WP3: Depth of analysis -- No unique solution. Multiple designs possible. Analysis needed to select the most appropriate one. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Rubrics |  | CLOs | Marks |  |
|  | Understands objectives, requirements, and constraints | WP1, WP2 | CLO 2 |  |
|  | Clearly explains the op-amp circuit principles for each stage. | WP1 | CLO 2 |  |
|  | Considers multiple solutions and selects suitable one based on analysis | WP1, WP3 | CLO 2 |  |
|  | Accurately calculates the circuit components. | WP1, WP3 | CLO 2 |  |
|  | Develops a final design which satisfies all requirements | WP1, WP3 | CLO 2 |  |
|  | Report |  |  |  |

Knowledge Profiles

WK1 - Natural Sciences: A systematic theory-based understanding of natural sciences applicable to the discipline.

WK2 - Mathematics and Computing: The concept-based mathematical thinking, numerical analysis, statistics and formal aspects of computer and information science to support analysis and modelling applicable to the discipline.

WK3 - Engineering Fundamentals: A systematic, theory-based formulation of engineering fundamentals required in an engineering discipline.

WK4 - Engineering Specialization: The knowledge of Engineering specialization that provides theoretical frameworks and bodies of knowledge for the accepted practice areas that are at the forefront in a discipline.

WK5 - Engineering Design: The Design Thinking Knowledge that supports engineering design in a practice area of an engineering discipline.

WK6 - Engineering Practice: The Knowledge of engineering practices (technology) in different practice areas of an engineering discipline.

WK7 - Engineering in Society: A systematic, comprehension-based knowledge of the role of engineers in a society and the professional issues related to practicing engineering profession in a discipline: ethics and the professional responsibility of an engineer to public safety including the impact of an engineering activity i.e. economic, social, cultural, environmental and sustainability

WK8 - Research Literature: Engagement with selected knowledge in the research literature of the discipline.